TU/CDOE

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION AUT 21 & SPR 22

DRE 201: ENERGY MANAGEMENT AND AUDITING

Time: 3 Hours                                                                             Total Marks: 70

The figures in the right-hand margin indicate marks for the individual question.

- Choose the right option                                                                                         2×10=20

- The sector with the lowest commercial energy consumption out of four major sectors in the country is

(a) Industry       					(b) Agriculture 
(c) Residential   					(d) Transport

(ii)	Energy manager should be well versed with

- Managerial and technical skills

(b) managerial and marketing skills

(c) Technical and economical analysis skills

(d) energy audit skills

(iii)	Which of the following is least important of a good energy information  system

(a) data accuracy        	 			(b) data relevance

(c) data volume          				(d) data validation

- The ratio of energy consumption to corresponding production is called

- specific energy consumption			(b) Energy intensity

(c) production energy ratio 			(d) specific production ratio

(v)	Energy supplied by combustion of fuel is equal to

(a) fuel mass x density

(b) fuel mass x specific heat

(c) fuel mass x calorific value

(d) fuel mass x heat capacity

- A three-phase induction motor is drawing 12 amps at 440 volts. If the operating power factor of the motor is 0.85, then the power drawn by the motor is

- 7.8 kW					(b) 4.5 kW

(c) 5.3 kW					(d) 23.4 Kw

P.T.O

- The sum of present values of all cash flows associated with a project called

- NPV						(b) PV

(c) Internal rate of return			(d) return on investment

- An energy audit requires

- Quantification of energy use

(b) qualification of energy use

(c) Converting all energy use to one single unit

(d) Reduction in power consumption

- A firm substitutes (replaces) expensive oil and starts firing wood in a boiler. The following scenario is most likely

- Higher energy consumption			(b) higher fuel cost

(c) better boiler efficiency			(d) less smoke in the stack

- Amount of oxygen (stoichiometric) needed for complete combustion of 1 kg of Methane

- 17.39 kg					(b) 4 kg

(c) 6 kg 						(d) None of the above

(a) 	Explain: Energy management, energy conservation and energy auditing.

- Explain the difference between energy conservation and energy efficiency and state one example where energy costs are reduced but energy consumption goes up.

- A cement industry has installed 6 MW DG set and is operated by furnace oil. The cost of furnace oil is Rs 35/liter. The average loading on the DG set is 5 MW and the hourly furnace oil consumption is 1230 liters. Estimate the cost of energy generation in Rs/kWh. The plant management is planning to convert the existing DG set to gas operated engine. The estimated cost of gas is Rs 20/Sm3. If the energy generation is 3.7 kWh/Sm3, calculate the generation cost per kWh with gas as fuel.

5+5+5=15

P.T.O

3.

(a)	Explain the use of sankey diagram.

- What is the significance of calculating specific energy consumption (SEC)?

- An ESCO is required to invest in a waste heat recovery project, which is expected to yield an annual saving of Rs.10, 00,000 and the life of the equipment is 7 years. If the ESCO expects a 20% IRR on this project, calculate the investment required to be made in the waste heat recovery project.

- Discuss four energy efficiency improvement opportunities in a boiler system.

5+5+6+4=20

- (a) 	Define power factor of an electrical gadget.

- How the power factor improvement is related with the energy conservation programme?

- A tube light is rated 40W, 230V and 50Hz, it draws a current of 0.3 amps. Calculate the (i) power factor and (ii) size of the capacitance bank for improving the existing power factor to 0.95.

5+4+6=15

*********