TU/CDOE

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (SPRING) 2021

DRE 104: WIND AND HYDRO ENERGY

Total Marks: 70		           Time: 3 hours

The figures in the right-hand margin indicate marks for the individual questions

------------------------------------------------------------------------------------

1. Choose the correct answer:                                                           2×10=20

- The share of hydro energy in India’s electricity installed capacity is-

(i) 12%				(ii) 26%

(iii) 33%				(iv) 45%

- The installed capacity of Three Gorges dam of China is-

(i) 22.5 GW				(ii) 18 GW

(iii) 32.5 GW				(iv) 12.5 GW

- What types of turbine would you recommend for 1000 m head?

(i) Kaplan				(ii) Propeller

(iii) Franics				(iv) Pelton

- The maximum power coefficient value for VAWT in compared to HWAT is-

(i) Lower 				(ii) Higher

(iii) No relation			(iv) Same

- The unit of scale parameter in Weibull distribution is

(i) No unit				(ii) m2/s

(iii) m/s				(iv) kg/m3

- Which factor is most important in available power output from wind?

(i)Density of Air		 	(ii) Hub height

(iii) Wind speed			(iv) Number of blades

- The value of maximum available power coefficient (CP) is

(i) 0.593		 		(ii) 0.798

(iii) 0.321				(iv) 0.213

P.T.O.

- How much power is available when wind speed is 6m/s

compared to when it was 3 m/s?

(i) About the same	                (ii) About twice as much

(iii) About four times as much	 (iv) About eight times as

much

- Multi-blade wind energy conversion systems are-

(i) More efficient than three blade system

(ii) Less efficient than three blade system

(iii)As efficient as two-or-three blade system

(iv)No effect on number of blades

- Pitch regulation is related to-

(i) Yaw mechanism		(ii) blade setting angle

(iii) Wind direction		(iv) Power output

2. Answer the following:

- What are the different forms of presenting wind data? How do you represent the wind data statistically?

- Define Betz limit and derive the value for CP (max).

- The wind regime at a site is described by Weibull scale parameter of 6m/s and shape parameter of 2. What is the total annual duration in hours for which a wind speed of 10m/s will be available at that site?

5+5+5=15

3. Answer the following:

- Draw a schematic diagram of a wind electric energy conversion system and discuss its all components.

- A wind generator with a 10m- diameter blade span has a cut-in wind speed (minimum speed for power generation) of 5 m/s, at which velocity the turbine generates 3 kW electric powers. Determine the efficiency of wind-turbine generator set.

- A wind turbine of 25 kW with rotor diameter of 10 m installed in an area of wind power density of 200 W/m2. Calculate the annual energy production in this site. Assume capacity factor is 0.25.

4. Answer the following:                                                              5+5+5+5=20

- How the electric power generated from hydro energy scheme?

P.T.O.

- What are the various factors considered in designing a hydroelectric power plant? Draw a schematic diagram of a hydropower plant. Explain in brief the various components of the power plant.

- Explain thermal-hydro mix in meeting the load demand of a certain region.

- A hydroelectric scheme has a catchment area of 204 km2 and the available runoff is 60% with annual rainfall of 180 mm. A head of 305 m is available on the average. Find the power that can be developed from the scheme if the overall efficiency of the plant is 85%.

***